To study Forward kinematics of 2 dof serial manipulators

Exp

No 3

Aim:  To study Forward kinematics of 2 dof serial manipulators

Instrument required:

Desktop PC with internet connection

Forward Kinematics:

Forward kinematics (FK) in robotics determines a robot's end-effector position and orientation (pose) in 3D space based on known joint parameters—such as angles for revolute joints or displacements for prismatic joints—using mathematical transformation matrices. It is essential for path planning, simulation, and controlling robot movements.

Step 1: Identify Home position:

- H Parameter Table:

| Frame | θi | di | ai | αi |
| --- | --- | --- | --- | --- |
| 10T | θ1 | 0 | L1 | 0 |
| 21T | θ2 | 0 | L2 | 0 |

10T   = Rot(Z, θ1).Trans(z,0).Trans(x.l1).Rot(x,0)

21T   =  Rot(Z,θ2).Trans(x,l2)

20T = 10T x 21T

Code for Matlab:

clc

clear

syms theta1 theta2 L1 L2

%theta1 = 0;

%theta2 = 0;

A = [cosd(theta1) -sind(theta1) 0 0;

sind(theta1) cosd(theta1) 0 0;

0 0 1 0;

0 0 0 1];

B = [1 0 0 L1;

0 1 0 0;

0 0 1 0;

0 0 0 1];

C = A* B

C =

[cos((pi*theta1)/180), -sin((pi*theta1)/180), 0, L1*cos((pi*theta1)/180)]
[sin((pi*theta1)/180), cos((pi*theta1)/180), 0, L1*sin((pi*theta1)/180)]
[ 0, 0, 1, 0]
[ 0, 0, 0, 1]

D = [cosd(theta2) -sind(theta2) 0 0;

sind(theta2) cosd(theta2) 0 0;

0 0 1 0;

0 0 0 1];

E = [1 0 0 L2;

0 1 0 0;

0 0 1 0;

0 0 0 1];

F=D*E

[cos((pi*theta1)/180)*cos((pi*theta2)/180) - sin((pi*theta1)/180)*sin((pi*theta2)/180), - cos((pi*theta1)/180)*sin((pi*theta2)/180) - cos((pi*theta2)/180)*sin((pi*theta1)/180), 0, L1*cos((pi*theta1)/180) + L2*cos((pi*theta1)/180)*cos((pi*theta2)/180) - L2*sin((pi*theta1)/180)*sin((pi*theta2)/180)]

[cos((pi*theta1)/180)*sin((pi*theta2)/180) + cos((pi*theta2)/180)*sin((pi*theta1)/180),   cos((pi*theta1)/180)*cos((pi*theta2)/180) - sin((pi*theta1)/180)*sin((pi*theta2)/180), 0, L1*sin((pi*theta1)/180) + L2*cos((pi*theta1)/180)*sin((pi*theta2)/180) + L2*cos((pi*theta2)/180)*sin((pi*theta1)/180)]

[ 0, 0, 1,0]

[0, 0, 0, 1]

Simlified:

Answer:

[1, 0, 0, L1 + L2]
[0, 1, 0, 0]
[0, 0, 1, 0]
[0, 0, 0, 1]

Result: