To study reverse Kinematics of 2 dof planer serial manipulator

Exp

No 4

Aim:  To study reverse Kinematics of 2 dof planer serial manipulator

Instrument required: Desktop PC with internet connection.

Inverse Kinematics:

Inverse kinematics (IK) in robotics is the mathematical process of calculating required joint angles and parameters to position a robot's end-effector (gripper, tool) at a specific Cartesian coordinate and orientation. It is the reverse of forward kinematics, allowing robots to move to desired locations

Derive the closed form IK of this 2 Dof manipulator.

Given Equation:

20T = 10T x 21T

Configuration Matrix:

Configuration matrix for above manipulator for theta1 =0 and theta2 = 0

Equating general equation with manipulator equation:

Solve for theta2:

For theta1:

Matlab Code:

clc;

clear;

Px = 9.1416; % end effector x

Py = 14.7984; % end effector y

L1 = 10; % link1 length

L2 = 8; % link2 length

D = (Px^2 + Py^2 - L1^2 - L2^2)/(2*L1*L2);

theta2 = atan2(sqrt(1-D^2), D);

theta1 = atan2(Py, Px) - atan2(L2*sin(theta2), L1 + L2*cos(theta2));

disp('Theta1 =')

disp(theta1)

disp('Theta2 =')

disp(theta2)

Answer:

Theta1 =
0.7854

Theta2 =
0.5236

Result: